- can esp32 control stepper motor high weight like 50 kgs, and one esp32 can control 3-4 motor simultaneously

- ChatGPT said:

Yes. One ESP32 can control 3–4 stepper motors simultaneously, including motors capable of moving a 50 kg load. But the ESP32 itself does not provide the power needed to move that load.

Basic architecture

┌──────────────┐

│    ESP32     │

│              │

│ STEP/DIR 1 ──┼──── Driver 1 ── Stepper 1

│ STEP/DIR 2 ──┼──── Driver 2 ── Stepper 2

│ STEP/DIR 3 ──┼──── Driver 3 ── Stepper 3

│ STEP/DIR 4 ──┼──── Driver 4 ── Stepper 4

└──────────────┘

│

Control signals

Separate high-current power supply

┌──────────────────┐

│ 24V / 36V / 48V │

└──────────────────┘

│    │    │    │

Driver Driver Driver Driver

The important part: 50 kg doesn't determine the motor size

You need to know how the 50 kg is being moved.

For example:

  - 50 kg vertically with a lead screw → stepper + appropriate gearbox/screw can work.

  - 50 kg on wheels → much less torque may be required.

  - 50 kg lifted by a pulley → calculate required torque based on pulley radius and gearing.

  - 50 kg moved horizontally on rails → friction and acceleration become important.

  - 50 kg rotating → inertia and required acceleration matter.

A 50 kg load could require a relatively small motor in one mechanism and a very large motor in another.

For 3–4 motors

The ESP32 is actually quite suitable for this. You can generate independent STEP/DIR signals for 4 drivers.

For example:

| Motor | ESP32 | Driver |
| --- | --- | --- |
| Motor 1 | STEP + DIR | TB6600 / DM542 |
| Motor 2 | STEP + DIR | TB6600 / DM542 |
| Motor 3 | STEP + DIR | TB6600 / DM542 |
| Motor 4 | STEP + DIR | TB6600 / DM542 |

For heavier loads, I'd generally avoid tiny A4988/DRV8825-style drivers and look at TB6600-class or better digital stepper drivers, depending on the motor current.

Do not power the motors from the ESP32. The ESP32 only sends the control signals. The motor drivers and a separate power supply provide the motor current.

If you tell me what the 50 kg load is doing (lifting, sliding, rotating, conveyor, CNC axis, etc.), the distance/speed, and whether one motor or all 4 motors share the 50 kg, I can calculate the approximate stepper motor torque, driver, power supply, and ESP32 wiring you need.

- You said:

see there will be system like input conveyor from where a product will come inside then that input conveyor will stop, then another motor of center which will rotate the product 360 and then it stop and then output conveyor will start and move part outside.

- ChatGPT said:

Yes — this is a very practical ESP32 application, and you don't necessarily need 4 motors. Your sequence sounds like a small automated indexing/inspection machine.

Your system

INPUT CONVEYOR

────────────────────>

│

▼

┌─────────────┐

│   CENTER    │

│   ROTATOR   │

│             │

│   Product   │

│     ↻       │  ← rotates 360°

│             │

└─────────────┘

│

▼

────────────────────>

OUTPUT CONVEYOR

The control sequence would be:

  - Input conveyor ON

  - Product enters the center station.

  - A sensor detects that the product is correctly positioned.

  - Input conveyor OFF

  - Center mechanism clamps/holds the product.

  - Center stepper rotates 360°

  - Center motor stops at the required position.

  - Clamp releases, if applicable.

  - Output conveyor ON

  - Product leaves the machine.

  - Output conveyor stops/continues depending on your next cycle.

  - Repeat.

ESP32 can easily coordinate this

You could have:

ESP32

│

┌──────────────┼──────────────┐

│              │              │

▼              ▼              ▼

Input conveyor   Center rotation  Output conveyor

motor            motor             motor

│              │                │

Driver 1       Driver 2          Driver 3

And additionally:

Sensors

│

├── Product entry sensor

├── Center-position sensor

├── Clamp sensor

└── Safety/E-stop

│

▼

ESP32

One important question: the 50 kg product

If the product itself is 50 kg, the center rotating mechanism is the part that needs careful engineering.

For example, if you're rotating a 50 kg object sitting on a bearing:

50 kg does not automatically mean you need a huge 50 kg-rated stepper.

The required motor torque depends on:

  - Diameter of the rotating platform

  - Where the product's center of gravity is

  - Bearing/friction

  - Whether the product is clamped

  - Desired rotation speed

  - Required acceleration/deceleration

  - Whether the product is balanced

  - Whether the motor directly drives the platform or uses a gearbox/belt/chain

A gear reduction can make a big difference.

For example:

50 kg PRODUCT

↻

┌──────────────────┐

│ ROTATING TABLE   │

└────────┬─────────┘

│

Bearing

│

┌──────▼──────┐

│ Gear/Belt   │

│ Reduction   │

└──────┬──────┘

│

Stepper Motor

For a 50 kg rotating product, I'd strongly consider a proper industrial stepper/servo + gearbox, rather than a small NEMA 17 motor.

Also, don't rely on "360°" alone

If the center station must rotate to a precise orientation, I'd recommend adding a home/zero sensor.

For example:

Start

│

▼

Home position

│

▼

Rotate 360°

│

▼

Slow down

│

▼

Stop at known position

If you need the product to rotate exactly 360° and return to the same orientation, a stepper can do that very well, but you should account for missed steps. For a machine handling a 50 kg product, an encoder or home/position sensor is a good idea.

I would design your machine roughly like this

ESP32
→ 3 motor drivers
→ Input conveyor motor
→ Center rotation motor
→ Output conveyor motor
→ Product sensors
→ Clamp actuator
→ Emergency-stop/safety circuit

And the ESP32 runs a simple state machine:

IDLE

↓

INPUT CONVEYOR RUNNING

↓

PRODUCT DETECTED

↓

INPUT STOP

↓

CLAMP PRODUCT

↓

ROTATE 360°

↓

ROTATION COMPLETE

↓

RELEASE CLAMP

↓

OUTPUT CONVEYOR RUNNING

↓

PRODUCT EXITED

↓

INPUT CONVEYOR RUNNING

Yes, one ESP32 can control all of this.

If you give me (1) product dimensions/weight, (2) rotating platform diameter, (3) how fast you want the 360° rotation, and (4) whether the product is sitting on the platform or hanging/clamped, I can size the center motor + gearbox + driver + power supply for your 50 kg system.

- You said:

alongwith these motors there will be 2 cameras unit. 1 on top including lighting, lesor, etc and second in right side which will capture images of that part from side view of 360. also it will not continue rotate 360, its depend on customer product type. like its required 4 side picture front, right, back, left. or 6 side corners also. so one esp32 can control everything efficiently,of an commercial inspection machine dynamically connected with software.

- ChatGPT said:

Yes — one ESP32 can handle the machine-control side efficiently, but for a commercial inspection machine I would not make the ESP32 responsible for the cameras, image processing, and software communication all by itself.

The best architecture is a two-level system:

COMMERCIAL INSPECTION MACHINE

│

┌───────────────┴───────────────┐

│                               │

MACHINE CONTROLLER                 PC / IPC

ESP32 / PLC                 Inspection Software

│                               │

┌───────┼────────┐              ┌───────┴────────┐

│       │        │              │                │

Motors   Sensors  Safety        Camera 1         Camera 2

│       │        │            TOP              SIDE

│       │        │              │                │

Input     Product   E-stop         Image            Image

conveyor  sensors                  processing       processing

Center

rotation

Output

conveyor

Your machine can work like this

Suppose the customer's product configuration says:

4 views: Front → Right → Back → Left

The software sends a job/configuration to the machine:

Product: ABC123

Views: 4

Angles: 0°, 90°, 180°, 270°

Top camera: ON

Side camera: ON

Lighting: Profile A

Then the machine controller executes:

1. Input conveyor → ON

2. Product detected

3. Input conveyor → STOP

4. Clamp → CLOSE

5. Rotation → 0°

6. Camera trigger → Capture

7. Rotate → 90°

8. Camera trigger → Capture

9. Rotate → 180°

10. Camera trigger → Capture

11. Rotate → 270°

12. Camera trigger → Capture

13. Clamp → OPEN

14. Output conveyor → ON

15. Product exits

16. Next product

For a 6-view inspection, the software could simply configure:

0°

60°

120°

180°

240°

300°

So you don't need to change the machine program every time.

This is where ESP32 is excellent

The ESP32 can act as the real-time machine controller.

It can control:

  - Input conveyor

  - Output conveyor

  - Rotation motor

  - Clamp

  - Lighting trigger

  - Camera trigger

  - Product sensors

  - Home sensor

  - Position sensors

  - Encoder

  - Emergency/safety inputs

  - Machine status

  - Communication with the inspection PC

And it can execute precise sequences without waiting for the PC.

But I would separate image processing

Don't do something like:

Camera

↓

ESP32

↓

Image processing

↓

Inspection decision

Instead:

ESP32

│

Machine sequencing

│

┌───────────┼───────────┐

▼           ▼           ▼

Motors      Sensors      Triggers

│

▼

┌───────────┐

│ Industrial│

│    PC     │

└─────┬─────┘

│

┌──────────┴──────────┐

▼                     ▼

Top Camera            Side Camera

│                     │

└──────────┬──────────┘

▼

Vision Software

│

┌──────────┴─────────┐

▼                    ▼

PASS                  FAIL

The PC can run OpenCV, a machine-vision SDK, AI models, database logging, customer recipes, UI, etc.

Very important: camera synchronization

For your application, I wouldn't simply tell the ESP32:

"Rotate motor 90°, wait 1 second, take picture."

A better industrial sequence is:

Rotate

↓

Reach target position

↓

Motor STOP

↓

Position stable?

↓

Lighting ON

↓

Camera trigger

↓

Image captured

↓

Lighting OFF

↓

Next angle

The camera exposure/trigger and lighting should be tightly synchronized.

This becomes particularly important if the side camera is capturing the rotating product.

Dynamic customer configuration

This is actually a very good use case for a recipe-based system.

Your software could have something like:

PRODUCT RECIPE

────────────────────────

Product: XYZ-500

Weight: 50 kg

Rotation: 4 views

View 1:   0°

View 2:  90°

View 3: 180°

View 4: 270°

Top inspection: YES

Side inspection: YES

Top lighting: Profile 3

Side lighting: Profile 2

Rotation speed: 20 RPM

Inspection timeout: 3 sec

Another customer could have:

PRODUCT RECIPE

────────────────────────

Product: ABC-200

Rotation: 6 views

0°

60°

120°

180°

240°

300°

Top inspection: YES

Side inspection: YES

The ESP32 doesn't need to know what the product means. It receives commands such as:

MOVE_TO_90

CAPTURE

MOVE_TO_180

CAPTURE

MOVE_TO_270

CAPTURE

The PC/software manages the recipe.

Communication

For a commercial machine, I would consider:

PC ↔ ESP32

  - Ethernet — my preferred option

  - RS-485/Modbus — very robust

  - CAN bus — excellent for machine control

  - USB — possible for development/smaller systems

I would generally prefer Ethernet or CAN/RS-485 over relying on Wi-Fi for critical machine control.

For example:

Inspection PC

│

Ethernet

│

▼

ESP32

│

├── Motor Driver 1

├── Motor Driver 2

├── Motor Driver 3

├── Camera Trigger

├── Lighting

├── Sensors

└── Clamp

One thing I would change from your original design

You mentioned 3–4 motors.

For the machine you've described, you may have:

  - Input conveyor motor

  - Center rotation motor

  - Output conveyor motor

  - Clamp actuator/motor

That's perfectly manageable for one ESP32.

But if this is going to become a commercial 50 kg industrial machine, I'd seriously consider whether the motor-control layer should ultimately be an industrial PLC/motion controller rather than ESP32, particularly if safety certification, 24/7 operation, maintenance, and customer-site reliability are important.

The ESP32 can absolutely be the prototype/control brain, and it can also be suitable in a production machine if the overall electrical/safety architecture is engineered properly.

The architecture I'd recommend for your project

┌───────────────────────────┐

│       HMI / SOFTWARE      │

│                           │

│ Product Recipes           │

│ Inspection Results        │

│ Images                    │

│ User Management            │

│ Reports / Database        │

└─────────────┬─────────────┘

│

Ethernet

│

┌─────────────▼─────────────┐

│      MACHINE CONTROLLER   │

│           ESP32            │

│                           │

│ Motion sequence           │

│ Position control          │

│ Sensors                   │

│ Camera triggers           │

│ Lighting                  │

│ Machine states            │

└──────┬─────┬─────┬────────┘

│     │     │

STEP  STEP  STEP

│     │     │

┌────▼┐ ┌──▼──┐ ┌▼────┐

│ IN  │ │ROT. │ │ OUT │

│MOTOR│ │MOTOR│ │MOTOR│

└─────┘ └─────┘ └─────┘

┌──────────────────────────┐

│      VISION SYSTEM       │

│                          │

│  TOP CAMERA + LIGHT     │

│  SIDE CAMERA + LIGHT    │

└────────────┬─────────────┘

│

▼

Inspection Engine

│

PASS / FAIL / DATA

So yes: one ESP32 can coordinate the entire mechanical sequence and dynamically handle 4, 6, 8, or other numbers of inspection angles. I'd keep the computationally heavy vision work on the PC/industrial computer.

If you're actually planning to build and sell this machine, I can help you design the complete architecture next — including ESP32 model, stepper/servo selection for 50 kg, motor drivers, encoder, sensors, camera interfaces, lighting control, Ethernet protocol, software ↔ ESP32 command structure, and the 4/6/8-angle dynamic recipe system.

- You said:

small plc controller suggestion for our machine esp32 instead

- ChatGPT said:

Yes. For a commercial inspection machine, I’d consider a small PLC instead of an ESP32 if your priority is industrial reliability, serviceability, and 24/7 operation.

My shortlist

| PLC | Best for | Motion | Communication | My take |
| --- | --- | --- | --- | --- |
| Siemens S7-1200 | Industrial/commercial machine | Good, with appropriate modules/drives | Ethernet, PROFINET | ⭐ Best established choice |
| CLICK PLUS | Cost-conscious machine | Basic motion/control | Ethernet, serial | ⭐ Good value |
| Schneider Modicon M221 | Compact machine | Pulse-train motion | Ethernet/Modbus | ⭐ Good option |
| Omron CP2E | Machine automation | Pulse outputs | Ethernet/serial depending on model | ⭐ Good |
| Unitronics | PLC + HMI in one | Good | Ethernet/serial | ⭐ Interesting for your machine |

For your particular machine, Siemens S7-1200 would be my first choice if the budget allows.

But there's an important distinction

I would not connect the cameras directly to the PLC and expect the PLC to process the images.

Instead:

INDUSTRIAL PC

┌──────────────────┐

│ Inspection       │

│ Software         │

│                  │

│ Product recipes  │

│ Image processing │

│ AI / OpenCV      │

│ Database         │

│ HMI              │

└────────┬─────────┘

│

Ethernet/PROFINET

│

┌────────▼─────────┐

│      PLC         │

│                  │

│ Sequence control │

│ Sensors          │

│ Motors           │

│ Camera triggers  │

│ Lighting         │

│ Clamp            │

└────────┬─────────┘

│

┌────────────┼─────────────┐

▼            ▼             ▼

Conveyor      Rotary        Conveyor

Motor        Motor          Motor

For your machine specifically

I'd probably use:

  - PLC: Siemens S7-1200 CPU 1212C/1214C class

  - Rotary axis: Stepper or, preferably for a 50 kg commercial machine, servo motor

  - Conveyor motors: Gear motors + appropriate drives

  - Rotation feedback: Encoder/home sensor

  - Product detection: Photoelectric sensor

  - Clamp: Pneumatic cylinder + solenoid, if appropriate

  - Cameras: Industrial USB3/GigE cameras connected to IPC

  - Lighting: PLC-controlled trigger/strobe controller

  - Inspection software: Industrial PC

  - PLC ↔ PC: Ethernet/PROFINET/OPC UA depending on the chosen architecture

One thing I'd strongly recommend

For the 50 kg rotating product, don't select the motor merely from the 50 kg number.

If you give me:

  - Product maximum weight = 50 kg

  - Maximum product diameter

  - Rotating platform diameter

  - Desired rotation time, e.g. 90° in 1 second

  - Whether the product is clamped

  - Whether rotation must be extremely precise

  - Approximate conveyor speed

I can calculate the required rotary torque + motor size + gearbox ratio, and then recommend a specific PLC + motor/servo + drive architecture rather than just choosing components by guesswork.

Log in for advice and recommendations tailored to you.

Log inSign up for free

- You said:

plc should be lower rate like 3000-4000 so i can prefer else i will choose esp32

- ChatGPT said:

Yes — if your target is ₹3,000–₹4,000 for the controller, I would not choose a Siemens S7-1200. It's overkill for your budget.

For your machine, I would look at low-cost compact PLCs in the ₹3k–₹5k range, but there is a catch: you need multiple pulse/STEP outputs if the PLC is going to control 3–4 stepper drives independently.

What I'd choose

For your application:

┌─────────────────────┐

│       PLC           │

│  24V industrial I/O │

└──────────┬──────────┘

│

┌────────────────┼────────────────┐

▼                ▼                ▼

Input conveyor     Rotary axis      Output conveyor

STEP/DIR          STEP/DIR          STEP/DIR

│                │                │

Driver           Driver           Driver

│                │                │

Motor            Motor            Motor

You also need inputs for:

  - Product sensor

  - Rotary home sensor

  - Clamp open/close sensors

  - Conveyor sensors

  - Emergency/safety status

  - Encoder, if used

And outputs for:

  - Motor drivers

  - Clamp

  - Camera trigger

  - Lighting trigger

  - Alarm/status

The problem with ₹3–4k PLCs

Many inexpensive PLCs advertise something like 8–16 I/O, but that doesn't mean all outputs can generate high-speed pulses.

For your machine, check specifically for:

High-speed pulse output / PTO / pulse + direction output

You ideally want 3 independent motion outputs.

If a cheap PLC has only one or two high-speed pulse outputs, it isn't a good replacement for your ESP32.

Honestly, ESP32 may be the better choice

For your particular machine, an ESP32 + proper industrial I/O interface can be a very capable solution at this budget.

I wouldn't connect 24 V sensors directly to the ESP32, though.

I'd build it like:

24V INDUSTRIAL POWER

│

├──────── Sensors

│

├──────── Motor Drivers

│

├──────── Solenoids

│

└──────── 24V → 5V/3.3V

│

ESP32

│

┌──────────────┼──────────────┐

│              │              │

STEP/DIR       STEP/DIR       STEP/DIR

│              │              │

Driver 1       Driver 2       Driver 3

│              │              │

Conveyor       Rotation       Conveyor

Use opto-isolated inputs/outputs, proper 24 V industrial power, fusing, contactors/relays where appropriate, and a separate hardwired safety/E-stop circuit.

That gives you something much closer to a PLC architecture while retaining the ESP32's low cost and flexibility.

My recommendation for your project

If this is:

Prototype / first machine / cost-sensitive → ESP32

Commercial machine deployed to many customers → low-cost PLC if you find one with sufficient PTO outputs

High-end industrial machine → Siemens/Omron/Schneider/etc.

And because you're already planning dynamic recipes (4 views, 6 views, different angles), two cameras, lighting, and PC software, the ESP32 actually gives you a lot of flexibility.

If you want, I can look up currently available ₹2,500–₹5,000 PLCs in India and find 3–5 models that have enough high-speed pulse outputs for your 3 motor system.